1. Release version and download file name

The newly released program version and download file name are as follows.

[Release Version]

| Model | Program Name | Current Ver. | New Ver. | Download file name | SHA-256 | Note |
| --- | --- | --- | --- | --- | --- | --- |
| C MF 385-1FS/ 385-1/ 335-1 | Controller firmware | A3GN30G0803-W99 | A3GN30G0851-W99 | C_MF385-1Series_Cont_A3GN30G0851-W99.exe | 14f0862bbb7a7d8b806a447782ce6446263a5048683bb8de766f0c56418f7774 | Supported for Web Browser |
|  |  | A3GN30G0803-899 | A3GN30G0851-899 | C_MF385-1Series_Cont_A3GN30G0851-899.exe | 10e5be53845c71f1ad5b014d0b51a7311e7593e66cc51afa8a4b6ec5bca42552 |  |

[Download file name]

C_MF385-1Series_Cont_A3GN30G0851-W99.exe

C_MF385-1Series_Cont_A3GN30G0851-899.exe

PRI_DLBT1615334EN_0.docx

2. Changes

The following has been supported.

- European regulations (ERP Lot 26 production)

The following issues have been fixed.

- SSL/TLS connection error occurs after Windows Update(KB3161639)[ERGRA_1600876403]

3. Restrictions

- Please set the following values in Switch No. 13 of Software Switch if you install this firmware.

Bit Assignment [00001000]  HEX Assignment [08]

- If the controller firmware in the machine is older than A3GN30Gx515-x99, when you upgrade it to this version of firmware, the startup screen continues to be displayed for a while after automatic reboot. It takes longer to become Ready than usual. If HDD encryption setting is set to ON, it takes even longer.

A moving image is shown on the startup screen.

Be sure to never turn off the main power until the machine restarts and becomes [Ready].

If you turn off the main power, HDD error will occur at the next startup. In order to resolve the HDD error, HDD formatting (HDD initialization) is required. HDD formatting deletes all the stored files in HDD so please be careful.

- OFF the setting of Wireless (AP mode) if it is ON, before upgrading or downgrading the Controller Firmware.

- Damages caused by downgrade from New version to Current version is not covered by the warranty.

4. Programs to be applied at the same time

3850/3350: 	Please use the Engine firmware Version [A3GN-0Y0-0050-G-00-1400] or later.

3850FS:		None.

5. How to install

When upgrading the Controller Firmware from the version [A3GN30G0206-999] or earlier to New version [A3GN30G0851-W99], please follow the procedure below.

1. Please install the Controller Firmware File name [C_MF385-1Series_Cont_A3GN30G0851-899].

2. Please install the Controller Firmware File name [C_MF385-1Series_Cont_A3GN30G0851-W99].

Please double-click the [exe] file and extract the [prn] file.

For detailed information, please refer to the Service manual [J. REWRITING OF FIRMWARE].

6. OpenAPI supported function level

The following is OpenAPI supported function level on this firmware.

| OpenAPI Version | 4.1K |
| --- | --- |

| Auth | 1.1/2.0/2.5/3.5/3.6/3.7/4.0/4.1 |
| --- | --- |
| Scan | 2.2/3.0/3.5/3.6/3.7/4.0 |
| PullPrint | 1.1/2.0/2.5/3.5/3.6/4.0 |
| Other A | 1.0/3.5/4.0 |
| Job Log | 1.0/3.5/3.6/4.0 |
| Setup | 1.3/2.0/2.1/2.5/3.5/3.6/3.7/3.8/4.0/4.1 |
| Browser (Auth) | 1.0/1.5/3.5/3.6/3.7/4.0/4.1 |
| Browser (Scan) | 1.0/1.1/3.5/3.6/3.7/4.0/4.1 |
| Browser (Pull) | 1.0/1.5/3.5/3.6/4.0/4.1 |
| Browser Launcher | 1.0/3.5/4.0 |
| Browser (General) | 4.0/4.1 |

7. IWS function level

| IWS function Version | 2.0.2 |
| --- | --- |

* Company names and product names in this document are the registered trademarks for respective companies.

Copyright(C) 2014 - 2016 KONICA MINOLTA, INC.